Tables and Figures.

Table 1. Protocol sequence to automate the on-line SPE- HPLC/UV for herbicides determination in corn samples.

Table 2. Analytical performance of the on-line SPE- HPLC/UV.

Table 3. Factorial design matrix (32) and the results obtained.

Table 4. Average recoveries (%) and relative standard deviations (RSD, %) from two corn samples fortified at 0.5 mg kg-1 level.

Table 1.

| Step | Description | MPV Position | IV Position | Volume (mL) | Flow rate (mL min-1) |
| --- | --- | --- | --- | --- | --- |
| 1 | Loading the sorbent conditioning solution for SPE column | 2 | 2-1, 4-3 | 1 | 5 |
| 2 | Dispense the sorbent conditioning solution1 into SPE column | 6 | 2-1, 4-3 | 1 | 1 |
| 3 | Loading water for washing sorbent | 3 | 2-1, 4-3 | 1 | 5 |
| 4 | Dispense water into the SPE for washing sorbent | 6 | 2-1, 4-3 | 1 | 1 |
| 5 | Loading sample in HC | 4 | 2-1, 4-3 | 10 | 5 |
| 6 | Dispense sample into SPE column | 6 | 2-1, 4-3 | 10 | 1 |
| 7 | Loading eluent in HC | 2 | 2-1, 4-3 | 0.5 | 5 |
| 8 | Dispense eluent into SPE column | 6 | 2-1, 4-3 | 0.5 | 1 |
| 9 | Start HPLC pump and detection | 6 | 6-1, 4-5 | 5 | 1 |
| 10 | Obtaining the chromatogram |  |  |  |  |
| 11 | Stop HPLC pump and detection | 6 | 6-1, 4-5 | 0 | 0 |
| 12 | The system is ready to start again |  |  |  |  |

Table 2.

| Parameter | 2,4-D | atrazine |
| --- | --- | --- |
| Lineal equation | y = 0.071x+0.0098 | y = 0.1647x+ 0.0018 |
| Correlation coefficient (r) | 0.9936 | 0.9990 |
| Lineal range (mg L-1) | 0.07-0.7 | 0.07-0.7 |
| RSD (%) | 5.7 (0.07 mg L-1) | 7.2 (0.07 mg L-1) |
|  | 6.0 (0.3 mg L-1) | 4.0 (0.3 mg L-1) |
| LOD (mg L-1) | 0.03 | 0.02 |
| LOQ (mg L-1) | 0.10 | 0.10 |
| Enrichment factor | 12.0 | 13.4 |

Table 3.

| Experiment | Methanol content in extractant (%) | Temperature (C) | 2.4-D recovery (%) | Atrazine recovery (%) |
| --- | --- | --- | --- | --- |
| 1 | - (30) | - (50) | 100.00 | 97.49 |
| 2 | 0 (50) | - (50) | 74.79 | 100.00 |
| 3 | + (70) | - (50) | 51.40 | 87.63 |
| 4 | - (30) | 0 (70) | 93.82 | 79.41 |
| 5 | 0 (50) | 0 (70) | 70.15 | 76.24 |
| 6 | + (70) | 0 (70) | 45.27 | 73.17 |
| 7 | - (30) | + (90) | 81.84 | 75.91 |
| 8 | 0 (50) | + (90) | 67.67 | 82.99 |
| 9 | + (70) | + (90) | 44.73 | 74.94 |
| 10 | 0 (50) | 0 (70) | 67.54 | 79.60 |
| 11 | 0 (50) | 0 (70) | 59.27 | 65.90 |

Table 4.

| Sample | Compound | Recovery (%) |
| --- | --- | --- |
| S1 | 2,4-D | 85.4 (4.2) |
|  | atrazine | 97.3 (3.1) |
| S2 | 2,4-D | 82.6 (2.3) |
|  | atrazine | 98.2 (3.4) |